Weather Station Chip - User Guide

1. Purpose

A compact, chip-based weather station for field use: two-button control, one 8-character LCD, and a 32.768 kHz system clock. Primary actions are: Mode to select what is shown, Start/Adjust to clear totals or adjust time, and Mode + Start/Adjust (within a short time window) for calibration and time setting.

2. What the Station Provides

Total rainfall (mm)

Instantaneous wind speed (m/s)

Wind direction (compass text on the 8x1 display)

Elapsed time

Time of day (24 h)

Optional: atmospheric pressure (hPa) and air temperature ( deg C)

3. Device Overview (high level)

Tipping-bucket rain gauge: pulse input (nRain, reed/switch)

3-cup anemometer: pulse input (nWind, reed/switch)

Wind vane via SPI to a 12-bit ADC (e.g. AD7466)

8-character x 1-line LCD

Mode and Start/Adjust (active low with pull-ups, per course pad conventions)

Demo input: when high, the time base runs faster for bench demos (see section 9)

Optional: barometric / temperature via SPI (e.g. MS5803) when that option is enabled

4. Installation and Good Use Practice

Mount in the open, away from walls and local obstructions.

Keep the rain gauge level and clear of debris.

For temperature, avoid local heat and direct sun on the sensor (when present).

After power-on, allow a short startup time before relying on the display (see project define start_up_time).

5. Mode Navigation

After reset, the first screen is Total rainfall.

Mode (single press, debounced in firmware): cycles the active measurement in this order:

Total rainfall -> Instantaneous wind speed -> Wind direction -> Elapsed time -> Time of day

then, if pressure/temperature is enabled: -> Pressure -> Temperature and back to Total rainfall.

If pressure/temperature is not enabled: from Time of day the next Mode press goes straight to Total rainfall.

Start/Adjust (single press): used for reset and for editing inside special modes (see section 7-8).

Buttons are debounced (~25 ms). Mode and Start/Adjust are distinguished from a chord by a ~100 ms window (dual-button detection).

6. Range, Resolution, and Accuracy (target / product; verify against calibration)

The table below is a user-facing target. Reported values also depend on installation, debouncing, and calibration (section 8). Low readings are often resolution-limited; at higher values, proportional error terms may dominate.

| Mode | Range (typ. display) | Resolution | Notes |
| --- | --- | --- | --- |
| Total rainfall | 0.0-999.9 mm (display saturates) | 0.1 mm | See section 7A; default scale from pulses |
| Wind speed | 0.0-99.9 m/s (display limit) | 0.1 m/s | Instantaneous from period between pulses, shows 0 if no pulse for 2 s |
| Wind direction | 16-point compass (or 3 characters on LCD) | N/A (sector step ~22.5 deg) | SPI + decode to letters |
| Elapsed time | 00:00-99:59 (hours:minutes on LCD) | 1 min | Internally counts seconds; long-term drift follows 1 Hz tick |
| Time of day | 00:00:00-23:59:59 | 1 s | User-set in adjustment sequence |
| Temperature / pressure | per sensor | per sensor | Only if option enabled |

7. How Each Function Works for the User

A) Total rainfall

Unit: mm, format like ddd.d mm on 8 characters.

Zero at reset.

Start/Adjust: a valid Start actuation clears the total (raw key path in the rain model). In practice, treat Start/Adjust as the control that clears the rain counter unless you are performing a two-button special action (see section 8).

Default sense (before calibration): each accepted rain pulse is scaled so 1 tip ~ 0.28 mm (see rain_gauge base conversion; four-digit calibration multiplies the result).

B) Instantaneous wind speed

Unit: m/s on the display.

Method: time between anemometer pulses; no pulse for 2 s -> show 0.0 m/s (default timeout).

Default sense: 1 tip/s ~ 0.67 m/s (based on 1.492 mi/h at 1 Hz; ~2.4 km/h per tick/s, matching the course-style assumption).

C) Wind direction

Vane and ADC as wired for your board; the chip shows right-aligned compass letters in the 8 characters (1-3 letters as needed).

Reference conditions (e.g. 3.3 V supply) should match your hardware.

D) Elapsed time

Display: In normal (non-Demo) operation, HH:MM (hours:minutes).

Zero at reset.

Start/Adjust: each single-Start press (not classified as a Mode+Start chord) clears the elapsed counter (see main_fsm / elapsed_time_counter).

Note: The same key also participates in other resets-see section 8.3 and Troubleshooting.

E) Time of day

24 h HH:MM:SS on the 8 characters (in normal, non-Demo).

Set as in section 8.1.

F) / G) Temperature and pressure (optional)

Shown only if the product image includes the pressure/temperature blocks.

Pressure is especially useful for trend; temperature for ambient air when the sensor is properly exposed.

8. User Adjustment Functions

8.1 Setting time of day

Go to Time of day (Mode).

Mode + Start/Adjust (both within the dual-button window) -> time set sub-modes.

Sub-modes, in order:

Set hours - each Start/Adjust advances hours (BCD, rolls past 23).

Mode -> Set minutes - Start/Adjust advances minutes.

Mode -> Set seconds - Start/Adjust does not count minutes up; it resets seconds (and sixtieths display fields) to 00.

Mode again (while on the seconds step) exits to normal Time of day view.

So: the third Mode press while in the sequence (from hours -> minutes -> seconds) returns to the clock; the seconds step uses Start/Adjust to zero the seconds, not to increment them.

8.2 Rain and wind calibration

From Total rainfall or Instantaneous wind speed, use Mode + Start/Adjust to enter the matching calibration page.

You step through four positions (shown as a multiplier ddd.d on the LCD, e.g. 1.035): Digit 3 (thousands) -> 2 -> 1 -> 0, then a further Mode press saves and returns to the same function you started from.

Start/Adjust in that screen increments the selected digit.

Behaviour detail: the rainfall and wind paths use the same "four digits + multiplier 1000 = 1.000x" style display; editing rules differ internally (rain uses stepped adds by digit weight; wind uses per-digit 0-9), but the user always sees the four-decimal place multiplier and the same "Mode advances digit / exit" pattern.

8.3 Single Start vs Mode+Start (important)

A short Start/Adjust without the Mode chord: resets elapsed time and (via the rain block) is also the way rain is cleared on a simple press. Plan usage if you need to clear one without the other: the chip may require separate wiring or a future software split-this document describes the current behavioural coupling.

A Mode + Start/Adjust within ~100 ms is treated as a chord for entering time set / calibration. The FSM is written so a chord does not issue the FSM's "single Start" elapsed clear; physical key interaction may still need care so you do not lose rain counts unintentionally when entering calibration.

Operational warning (important): Start/Adjust is shared across elapsed reset, rain clear, and adjustment increments.

To avoid unintended clears, do not press Start/Adjust before Mode+Start chord is recognized.

Recommended sequence: first press and hold Mode, then press Start/Adjust within the dual-button window.

9. Demonstration mode (Demo high)

With Demo asserted:

The 1 Hz tick is replaced by a faster tick. With the default 32.768 kHz core clock, the divider is /64 relative to a real-time second, so the time-related behaviour is 64x faster, not 60x.

Elapsed time on the LCD switches to MM:SS (minutes:seconds) instead of HH:MM.

Time of day shows MM:SS:xx (minutes, seconds, sixtieths-style field), and this display wraps after 24 "demo minutes" (same as a full 24 h in compressed form for the exercise).

Rain, wind, and direction are not sped up by Demo; only the time bases and time displays that use the divided clock reflect the faster tick.

10. Error and Limitations (summary)

Reed bounce and monostable debounce (~25 ms) affect rain/wind.

ADC and analog tolerances for direction.

Clock tolerance (e.g. crystal spec) affects long-term time.

Display rounding and calibration multipliers.

Demo must be off for real time-of-day and elapsed time.

Recommended acceptance criteria (set per product calibration report):

- Rainfall total error: define as +/-X% or +/-Y mm over a stated accumulation interval.

- Wind speed error: define as +/-X% of reading or +/-Y m/s, whichever is greater.

- Time base drift: define as +/-X s/day at nominal temperature with Demo off.

11. Quick start

Mount for open exposure.

Power and reset; confirm Total rainfall first.

If needed, set Time of day (section 8.1).

Optional: rain/wind calibration (section 8.2).

Log readings with where and when; use pressure trend (if available).

12. Troubleshooting

| Symptom | What to check |
| --- | --- |
| No button response | Wiring, active-low sense, and external pull-up (if used on your board) |
| Rain/wind wrong | Reed alignment, cable, and demonstration mode off for timing tests |
| Direction unstable / blank | Vane, SPI, supply, and ADC reference |
| Time wrong | 32.768 kHz source, reset, and not in Demo when expecting real time |
| Time too fast | Demo input high (64x with default clock options) |
| Unwanted clears on Start | Remember Start is shared for rain, elapsed, and increments in adjustment modes |